COMP 3980

Data Comm Assignment 4 Design Document

An implementation of the Grapefruit Protocol

Lewis Scott - A00855385

Christofer Klassen - A00879060

Jeff Bayntun - A00860994

Thomas Tallentire - A00882367

Table of Contents

State Flow Diagram	2

Pseudo Code	3

Setup	3

Idle	3

Receive Start	3

Receive Start	4

Send Start	5

Send Data	5

# State Flow Diagram

# Pseudo Code

## Setup

Function setupComm() {

Open the Comm Port with pre-defined settings

If Comm Port is not open

Display an error message

Terminate program

Create a Communications thread

}

## Idle

Function idle() {

Create a Receiving thread

While true

If not receiving

If buffer > 0 and resetTimer = 0

Sending <- true

sendSync<- SYN1

sendControlCharacter( ENQ )

waitForEnqResponse()

}

## Receive Start

Function waitForEnquiry() {

While true

If not sending

Wait infinitely for ENQ

If ( ENQ received )

Receiving <- true

receiveSync<- SYN1

ENQReceived++

sendControlCharacter( ACK )

ACKSent++

waitForAckResponse()

}

## Receive Data

Function waitForAckResponse() {

While ( receiving )

If ( wait for response = timeout )

Receiving <- false

Else

ACKreceived++

validatePacket( response )

}

Function validatePacket( response ) {

crcValid<- checkCRC( response )

if ( crcValid and receiveSync = response sync )

ValidPacketsReceived++

if ( response header = EOT )

receiving<- false

receiveSync<- not receiveSync

receiveBuffer<- receiveBuffer + trimResponse( response )

displayBuffer()

else

InvalidPacketsReceived++

NAKSent++

sendControlCharacter( NAK )

}

Function trimResponse( response ) {

I <- 2

If ( response header = EOT )

While ( response[i] is not ETX and i<1018)

I <- I + 1

Return substring of response from 2 to i

Else

Return substring of response from 2 to 1018

}

## Send Start

Function waitForEnqResponse() {

if ( wait for response = timeout )

sending<- false

else

sendData()

resetTimer<- random( MIN_RESET to MAX_RESET )

}

## Send Data

Function sendData() {

Misses <- 0

Sent <- 0

While ( buffer> 0 and misses < MAX_MISSES and sent < MAX_SENT )

{

constructPacket( packet, maxSends)

dataSize<- min( bufferSize, 1019 )

Put packet through port

Response <- waitForSendResponse()

If ( response = ACK ) {

ACKReceived++

PacketsSentSuccessfully++

Sent <- sent + 1

sendSync<- not sendSync

Remove dataSize bits from buffer

} else {

Misses <- misses + 1

NAKReceived++

PacketsSentUnsuccessfully++

}

}

}

Function constructPacket( packet, maxSends ) {

If ( buffer> 1018 andnot maxSends )

Packet <- packet + ETB

syncSend<- not syncSend

Packet <- packet + syncSend

For I <- 0 to 1017

Packet <- packet + buffer[i]

Else

Packet <- packet + EOT

syncSend<- not syncSend

Packet <- packet + syncSend

For I <- 0 to size of buffer

Packet <- packet + buffer[i]

Packet <- packet + ETX

For I <- 0 to 1017 – size of buffer – 1

Packet <- packet + ETX

setCRC( packet )

}

Function setCRC( packet ) {

Crc<- Create CRC with dividend 04C11DB7

Packet <- packet + crc

}